Crystal Structure Report for J. Kovacs

by Maike Blakely & Werner Kaminsky

Sample Submitted by Alex Downing

ID – AD1028

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 7/14/2017

Documentation finished: 8/7/2017

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A red prism needle, measuring 0.15 x 0.07 x 0.07 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 40 mm and exposure time was 240 seconds per frame for all sets.  The scan width was 1o.   Data collection was 97.5% complete to 25o  in .  A total of  18732 reflections were collected covering the indices, -14<=h<=14, -11<=k<=11, -15<=l<=15.  2422 reflections were symmetry independent and the elevated Rint = 0.1799 reflects the small size of the sample.  Indexing and unit cell refinement indicated a primitive monoclinic lattice.  The space group was found to be P2/c  (No. 13).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXS, SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure is of good quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the molecular unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. Hydrogen, anion, and disorder were removed for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C17 H33 F6 Fe N3 O P S2

Formula weight 	560.40

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P 2/c

Unit cell dimensions	a = 11.349(8) Å	= 90°.

b = 9.077(6) Å	= 110.88(4)°.

c = 12.448(9) Å	 = 90°.

Volume	1198.1(15) Å3

Z	2

Density (calculated)	1.553 Mg/m3

Absorption coefficient	0.932 mm-1

F(000)	582

Crystal size	0.15 x 0.07 x 0.07 mm3

Theta range for data collection	1.921 to 26.451°.

Index ranges	-14<=h<=14, -11<=k<=11, -15<=l<=15

Reflections collected	18732

Independent reflections	2422 [R(int) = 0.1799]

Completeness to theta = 25.00°	97.5%

Max. and min. transmission	0.7454 and 0.6088

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	2422 / 83 / 213

Goodness-of-fit on F2	1.012

Final R indices [I>2sigma(I)]	R1 = 0.0879, wR2 = 0.1854

R indices (all data)	R1 = 0.1936, wR2 = 0.2363

Largest diff. peak and hole	0.657 and -0.569 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,   Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2013) Crystal structure refinement with SHELXL. Acta Cryst. (2015). C71, 3-8.

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A. (1997) MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland,.

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565