Crystal Structure Report for J. Kovacs

by Maike Blakely & Werner Kaminsky

Sample Submitted by B. Leipzig

ID – BKL_A_088

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data Collection  started: 7/19/2016

Documentation finished: 7/22/2016

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A colorless prism measuring 0.03 x 0.01 x 0.01 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 40 mm and exposure time was 240 seconds per frame for all sets.  The scan width was 1.0o.   Data collection was 99.3% complete to 25o  in .  A total of  13868 reflections were collected covering the indices, h = -9 to 9, k = -13 to 13,  l = -17 to 17.  4521 reflections were symmetry independent and the Rint = 0.1009 indicated that the data was appropriate.  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be P -1  (No.2).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXS, SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure was found to be a dimer bridged by two chlorine atoms. Two hexafluorophosphate anions balance the charge of the dimer, crystallized together with two acetonitrile solvent molecules.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level; solvent and anions omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula                	 C17 H25 Cl F6 Mn N7 P S

Formula weight                    	594.86

Temperature                       	100(2) K

Wavelength                        	0.71073 A

Crystal system, space group       	Triclinic,  P -1

Unit cell dimensions             	 a = 7.8232(12) A   alpha = 70.691(9) °

b = 11.4274(16) A    beta = 88.291(9) °

c = 14.851(2) A   gamma = 82.748(10) °

Volume                            	1242.8(3) Å-3

Z, Calculated density            	 2,  1.590 Mg/m^3

Absorption coefficient            	0.852 mm^-1

F(000)                            	606

Crystal size                     	 0.30 x 0.10 x 0.10 mm

Theta range for data collection   	1.453 to 25.351 °

Limiting indices                  	-9<=h<=9, -13<=k<=13, -17<=l<=17

Reflections collected / unique    	13826 / 4521 [R(int) = 0.1009]

Completeness to theta = 25.000    	99.5 %

Goodness-of-fit on F2            	1.001

Final R indices [I>2sigma(I)]     	R1 = 0.0602, wR2 = 0.1154

R indices (all data)              	R1 = 0.1268, wR2 = 0.1369

Largest diff. peak and hole      	 0.691 and -0.429 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,   Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2013) Crystal structure refinement with SHELXL. Acta Cryst. (2015). C71, 3-8.

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A. (1997) MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland,.

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565