Crystal Structure Report for A Velian

by Paige Gannon & Werner Kaminsky

Sample Submitted by Ben Mitchell

ID – BSM 678

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data Collection  started: 4/12/2022

Documentation finished: 4/15/2022

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A brown needle, measuring 0.26 x 0.05 x 0.05 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 120 seconds per frame for all sets.  The scan width was 1o.   Data collection was 99.8% complete to 25o in .  A total of  24488 merged reflections were collected covering the indices, -64<=h<=63, -17<=k<=17, -52<=l<=52 with  estimated Rint = 0.2445. Indexing and unit cell refinement indicated a C-centered monoclinic lattice.  The space group was found to be C2/c  (No. 15).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The asymmetric unit contains one-and-a-half Fe-dimers. Two irons are bridged by an oxygen. The half dimer is completed via symmetry. Several benzene solvent molecules were found, but several others are present in addition but disordered including some pentane. The contribution of disordered solvent to the diffraction pattern was removed with SQUEEZE.

The crystal was small, data fairly weak. Thus, some restraints were required to stabalize the displacement parameters of a few atoms via DELU and all carbon nitrogen and oxygen atoms via ISOR. Phenyl moieties were idealized via AFIX 66 loops and similar Fe[P(phen)2NtolSe]2 groups were related via SAME commands. 58 bad reflections were OMIT-ted from the refinement.

The structure is moderate quality possibly publishable with some disclaimer. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. Solvent omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C88 H80 Fe2 N4 O P4 Se4

Formula weight 	1760.98

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	C 2/c

Unit cell dimensions	a = 53.32(2) Å	a= 90°.

b = 14.104(6) Å	b= 122.515(8)°.

c = 43.80(2) Å	g = 90°.

Volume	27776(22) Å3

Z	12

Density (calculated)	1.263 Mg/m3

Absorption coefficient	1.997 mm-1

F(000)	10704

Crystal size	0.260 x 0.050 x 0.050 mm3

Theta range for data collection	1.513 to 25.027°.

Index ranges	-64<=h<=63, -17<=k<=17, -52<=l<=52

Reflections collected	24488

Independent reflections	24488 [R(int) = 0.2445]

Completeness to theta = 25.000°	99.8 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	24488 / 1735 / 1134

Goodness-of-fit on F2	1.020

Final R indices [I>2sigma(I)]	R1 = 0.1329, wR2 = 0.2906

R indices (all data)	R1 = 0.3204, wR2 = 0.3879

Largest diff. peak and hole	1.247 and -1.691 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565