Crystal Structure Report for A. Velian

by Sebastian Krajewski &Werner Kaminsky

Sample Submitted by Daniel Zhou

ID – DZ402

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 11/28/2022

Documentation finished: 12/1/2022

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A brown prism, measuring 0.29 x 0.09 x 0.08 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 30 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 99.9% complete to 25o in .  A total of 48704 reflections were collected covering the indices, -58<=h<=58, -24<=k<=24, -36<=l<=36.  24785 reflections were symmetry independent and the Rint = 0.0614 indicated that the data was of good quality.  Indexing and unit cell refinement indicated a C-centered monoclinic lattice.  The space group was found to be C 2/c  (No. 15).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The Fe3-Fe3’- bridging azide between two clusters as well as terminal azide on Fe2 appear to be disordered. Additional toluene disorder is observed and a liquid solvent toluene, identified by squezze analysis, accompanies one cluster dimer. The contribution of that solvent to the diffraction pattern was removed with SQUEEZE, while one ether molecule per cluster was identified and refined. Disordered toluenes were related via SAME and optimised with AFIX 66 and in one case FLAT commands. Some close or overlapping disordered atoms from toluene and azides had their displacement parameters linked via EADP cards and convergence of the anisotropic ellipsoids was helped with ISOR for those moieties. 17 ‘bad’ diffraction peaks were explicitely omitted from the refinement.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. Disorder omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C236 H224 Co12 Fe6 N21 O2 P12 Se16

Formula weight 	6063.61

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	C 2/c

Unit cell dimensions	a = 46.768(3) Å	a= 90°.

b = 19.4665(15) Å	b= 114.629(3)°.

c = 28.928(2) Å	g = 90°.

Volume	23940(3) Å3

Z	4

Density (calculated)	1.682 Mg/m3

Absorption coefficient	3.727 mm-1

F(000)	12028

Crystal size	0.290 x 0.090 x 0.080 mm3

Theta range for data collection	1.150 to 26.549°.

Index ranges	-58<=h<=58, -24<=k<=24, -36<=l<=36

Reflections collected	48704

Independent reflections	24785 [R(int) = 0.0614]

Completeness to theta = 25.000°	99.9 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	24785 / 440 / 1458

Goodness-of-fit on F2	1.063

Final R indices [I>2sigma(I)]	R1 = 0.0620, wR2 = 0.1447

R indices (all data)	R1 = 0.1266, wR2 = 0.1827

Largest diff. peak and hole	1.770 and -1.709 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565