Crystal Structure Report for B. Cossairt

by Werner Kaminsky & Sebastian Krajewski

Sample Submitted by

Emily Miura-Stempel

ID – EMS-III-012C

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 9/30/2024

Documentation finished: 10/18/2024

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A yellow prism, measuring 0.20 x 0.15 x 0.11 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 45 mm and exposure time was 10 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 100% complete to 25o in .  A total of  85691 merged were collected covering the indices, -34<=h<=34, -33<=k<=33, -35<=l<=35.  22074 reflections were symmetry independent and the Rint = 0.0334 indicated that the data was of excellent quality (average 0.07).  Indexing and unit cell refinement indicated a C-centered monoclinic lattice lattice.  The space group was found to be C2/m   (No. 12).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The contribution of disordered solvent to the diffraction pattern was removed with SQUEEZE.

The asymmetric unit consists of two cluster halves, completed by symmetry. The clusters contain several disordered carboxylates which were refined with appropriate restraints.

Hydroxyl groups were identified by their long Ce-O distances (O4, O12, O18, O36, O39). The protons were constraint at 1Å towards the oxygen.

The admixture of Yb was tested by refining the cite occupancies of Ce/Yb with equal location and displacement parameters. Only a few ‘pairs’ allowed refinement convergence, i.e. Ce11, Ce12, Ce16, Ce20, Ce22 with occupancies 0.91, 0.95, 0.99, 0.94, 0.95, those of Yb are 1- those shown.

The structure is of appropriate quality and with some commends of its limitations ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the asymmetric unit with thermal ellipsoids at the 50% probability level. Disorder omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C296 H452 Ce75.14 N16 O268 Yb0.87

Formula weight 	19200.30

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	C 2/m

Unit cell dimensions	a = 29.141(4) Å	a= 90°.

b = 28.145(4) Å	b= 92.294(9)°.

c = 29.789(4) Å	g = 90°.

Volume	24413(6) Å3

Z	2

Density (calculated)	2.612 Mg/m3

Absorption coefficient	7.100 mm-1

F(000)	17805

Crystal size	0.200 x 0.150 x 0.110 mm3

Theta range for data collection	1.006 to 25.027°.

Index ranges	-34<=h<=34, -33<=k<=33, -35<=l<=35

Reflections collected	85691

Independent reflections	22074 [R(int) = 0.0334]

Completeness to theta = 25.027°	100.0 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	22074 / 4259 / 1863

Goodness-of-fit on F2	1.083

Final R indices [I>2sigma(I)]	R1 = 0.0387, wR2 = 0.0868

R indices (all data)	R1 = 0.0537, wR2 = 0.1002

Largest diff. peak and hole	2.953 and -3.444 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565