Crystal Structure Report for D. Reed

by Werner Kaminsky

Sample Submitted by Jay Lee

ID – KL_01_143_EtOAc

X-ray Crystallography Laboratory

University of Washington

28. July 2026

Data Collection  started: 10/11/2023

Documentation finished: 10/19/2023

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A green block, measuring 0.15 x 0.15 x 0.10 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 45 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 98.5% complete to 25o in .  A total of 140917 merged reflections were collected covering the indices, -38<=h<=37, -38<=k<=38, -93<=l<=93.  13447 reflections were symmetry independent and the Rint = 0.1482 indicated that the data was of appropriate quality.  Indexing and unit cell refinement indicated a trigonal lattice.  The space group was found to be R3c (No. 167).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure is pseudo-cubic but solved in a humangously large trigonal lattice. The asymmetric unit contains 1/6 of the total molecule.DMA and other solvent as well as the ligand to the complex were diorderd. The contribution of additional disordered solvent to the diffraction pattern was removed with SQUEEZE.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 25% probability level. Disorder and solvent omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C258 H204 Cu12 N12 O60

Formula weight 	5194.80

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Trigonal

Space group 	R -3 c :H

Unit cell dimensions	a = 31.5142(17) Å	a= 90°.

b = 31.5142(17) Å	b= 90°.

c = 77.192(6) Å	g = 120°.

Volume	66392(9) Å3

Z	6

Density (calculated)	0.780 Mg/m3

Absorption coefficient	0.609 mm-1

F(000)	15984

Crystal size	0.150 x 0.150 x 0.100 mm3

Theta range for data collection	0.914 to 25.462°.

Index ranges	-38<=h<=37, -38<=k<=38, -93<=l<=93

Reflections collected	140917

Independent reflections	13447 [R(int) = 0.1482]

Completeness to theta = 25.242°	98.5 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	13447 / 811 / 630

Goodness-of-fit on F2	1.005

Final R indices [I>2sigma(I)]	R1 = 0.0801, wR2 = 0.2333

R indices (all data)	R1 = 0.1309, wR2 = 0.2783

Largest diff. peak and hole	0.994 and -0.536 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565