Crystal Structure Report for D. Reed

by Werner Kaminsky

Sample Submitted by Jay Lee

ID – KL_01_152

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data Collection  started: 10/18/2023

Documentation finished: 10/24/2023

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A clear light blue prism, measuring 0.10 x 0.07 x 0.05 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 120 seconds per frame for all sets.  The scan width was 0.75o.   Data collection was 98% complete to 25o in .  A total of 72378 merged reflections were collected covering the indices, -26<=h<=26, -27<=k<=26, -27<=l<=27, Rint = 0.1225.  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be R1 (No. 2).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure exhibits some disorder causing diffraction intensities to drop rapidly with increased diffraction angles and some heavily disordered DMF solvent was treated with  SQUEEZE which removes the contribution of electron density in voids from the diffraction pattern.

1/2 of the cage molecule is found in the asymmetric unit from which the whole molecule is developed via symmetry operations.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. Disorder omitted for clarity; some solvent SQUEEZEd.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C270 H214 Cu12 N18 O62

Formula weight 	5465.06

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Triclinic

Space group 	P -1

Unit cell dimensions	a = 21.9150(13) Å	a= 79.157(3)°.

b = 22.5813(12) Å	b= 72.056(3)°.

c = 22.7106(13) Å	g = 68.851(3)°.

Volume	9934.4(10) Å3

Z	1

Density (calculated)	0.913 Mg/m3

Absorption coefficient	0.682 mm-1

F(000)	2804

Crystal size	0.100 x 0.070 x 0.050 mm3

Theta range for data collection	1.032 to 25.472°.

Index ranges	-26<=h<=26, -27<=k<=26, -27<=l<=27

Reflections collected	72378

Independent reflections	36567 [R(int) = 0.1225]

Completeness to theta = 25.242°	99.8 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	36567 / 2700 / 1818

Goodness-of-fit on F2	1.087

Final R indices [I>2sigma(I)]	R1 = 0.1250, wR2 = 0.2568

R indices (all data)	R1 = 0.2365, wR2 = 0.3055

Largest diff. peak and hole	1.543 and -1.237 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,   Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565