Crystal Structure Report for D. Reed

by Sebastian Krajewski and

Werner Kaminsky

Sample Submitted by Jay Lee

ID – MK_6_7A

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data Collection  started: 3/6/2025

Documentation finished: 3/20/2025

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A green block, measuring 0.25 x 0.19 x 0.18 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 60 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 99.6% complete to 25o in .  A total of  38969 merged reflections were collected covering the indices, 0<=h<=34, 0<=k<=34, -63<=l<=64.  19741 reflections were symmetry independent and the Rint = 0.2183 indicated that the data was challenging.  Indexing and unit cell refinement indicated a body centered tetragonal lattice.  The space group was found to be I 4/m  (No. 87).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The molecule exhibits massive disorder and contains liquid solvent. As a result, diffuse scattering is high and high theta angle diffraction data becomes invisible, regardles of exposure time and beam intensity. Thus high angle diffraction data is mostly missing.

The full molecule is generated from the content of the asymmetric unit via four symmetry operations. Several CO2 groups are brought into bonding to Cu via space group symmetries, thus, symmetry  operations are needed to get some Oxygen atoms into a bonding position.

The contribution of some disordered solvent to the diffraction pattern was removed with SQUEEZE

The structure is of lower quality and the explanation given above should be given in the supplementary if being published. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 5% probability level. Hydrogen atoms, disorder and solvent omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C696 H552 Cu24 N24 O120

Formula weight 	12696.54

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Tetragonal

Space group 	I 4/m

Unit cell dimensions	a = 28.5637(17) Å	a= 90°.

b = 28.5637(17) Å	b= 90°.

c = 53.726(4) Å	g = 90°.

Volume	43834(6) Å3

Z	2

Density (calculated)	0.962 Mg/m3

Absorption coefficient	0.625 mm-1

F(000)	13104

Crystal size	0.250 x 0.190 x 0.180 mm3

Theta range for data collection	0.758 to 25.119°.

Index ranges	0<=h<=34, 0<=k<=34, -63<=l<=64

Reflections collected	38969

Independent reflections	19741 [R(int) = 0.2183]

Completeness to theta = 25.119°	99.6 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	19741 / 2608 / 1217

Goodness-of-fit on F2	0.881

Final R indices [I>2sigma(I)]	R1 = 0.1347, wR2 = 0.3330

R indices (all data)	R1 = 0.3535, wR2 = 0.4282

Largest diff. peak and hole	0.959 and -0.734 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565