Crystal Structure Report for G. Khalil

Sample ID - PtTFPP

by Werner Kaminsky

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data backup accessibly through ftp:

computer: 		chmraypc.cpac.washington.edu

login: 		anonymous

password:		(your name or email address)

location: 	c:\structures\khalil\PtTFPP

Data backup accessibly through http:

URL: 	http://cad4.cpac.washington.edu/structures

Username: 	xray-user

Password: 	hkl-data

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@wintensor.com

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A red rism, size 0.14 x 0.14 x 0.12 mm was mounted on a glass capillary with oil.  Data was collected at -143oC with one set of exposure as follows:

Scan 		Type	Scanlength (deg.)  start (deg.)	     		     	

1	      	        	157.0					3.70	-51.5	160  	 0

Crystal-to-detector distance was 30 mm and exposure time was 30 seconds per degree for all sets.  The scan width was 2o.   Data collection was 30.5% complete to 25.35o in .  A total of  28581 partial and complete reflections were collected covering the indices, h = -23 to 23, k = -23 to 11,  l = -28 to 25.  3139 reflections were symmetry independent and the Rint = 0.1202 indicated that the data was of slightly less than average quality (0.07).

Indexing and unit cell refinement indicated a monoclinic P lattice.  The space group was found to be R3 (No.148).

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S = exp(2B(sinq/)2)/ scale

S is calculated from the scale and the B factor is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

No Solution was capable of producing a complete heavy atom phasing model. However, inserting the modofied solution for a similar compound (H2TFPP) resulted in a model consistent with the proposed structure which refined well with SHELXL97.

All hydrogen atoms were located using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.