Crystal Structure Report for

Rupam Dinda

by Werner Kaminsky

ID – SL_5

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 4/16/2018

Documentation finished: 4/19/2018

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures. Even I  have to pay for my own structures for my research.

A red prism, measuring 0.25 x 0.10 x 0.07 mm3 was mounted on a loop with oil.  Data was collected at -143oC on a Nonius Kappa CCD FR590 single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 30 mm and exposure time was 200 seconds per degree for all sets.  The scan width was 1.6o.   Data collection was 99.9% complete to 25o  in .  A total of  8938 merged reflections were collected covering the indices, -18<=h<=18, -9<=k<=9, -25<=l<=25.  4677 reflections were symmetry independent and the Rint = 0.0841 indicated that the data was close to average quality (0.07).  Indexing and unit cell refinement indicated a primitive monoclinic lattice.  The space group was found to be P 21/c (No. 14).

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S is calculated from the scale and the B factor determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SHELXS, SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

A disordered water bridges ketones O3 with O3’ from a symmetry related molecule.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C52 H38 N8 O9 V2

Formula weight 	1020.78

Temperature 	294(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P 21/c

Unit cell dimensions	a = 14.5422(6) Å	= 90°.

b = 7.9551(5) Å	= 105.123(4)°.

c = 20.5708(9) Å	 = 90°.

Volume	2297.3(2) Å3

Z	2

Density (calculated)	1.476 Mg/m3

Absorption coefficient	0.476 mm-1

F(000)	1048

Crystal size	0.250 x 0.100 x 0.070 mm3

Theta range for data collection	2.182 to 26.368°.

Index ranges	-18<=h<=18, -9<=k<=9, -25<=l<=25

Reflections collected	8938

Independent reflections	4677 [R(int) = 0.0841]

Completeness to theta = 25.000°	99.9 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	4677 / 5 / 332

Goodness-of-fit on F2	1.063

Final R indices [I>2sigma(I)]	R1 = 0.0645, wR2 = 0.1200

R indices (all data)	R1 = 0.1497, wR2 = 0.1534

Largest diff. peak and hole	0.314 and -0.771 e.Å-3

Otwinowsky, Z.; Minor, W. In: Methods in Enzymology; Carter, C. W., Jr., Sweet, R. M., Eds.; Academic Press: New York, 1997, 276, 307-326.

(a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.

Sheldrick, G. M. SHELXL-97: Program for the Refinement of Crystal Structures 1997 University of Gottingen, Germany.

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.

Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.

Farrugia, L. J. Appl. Cryst. 1997, 30, 565