Crystal Structure Report for A. Velian

by Werner Kaminsky

Sample Submitted by

Sebastian Krajewsky

ID – SMK10b

X-ray Crystallography Laboratory

University of Washington

28. July 2026

Data Collection  started: 9/18/2020

Documentation finished: 9/23/2020

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A tiny black prism, measuring 0.10 x 0.05 x 0.04 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Nonius Kappa CCD FR590 single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 41 mm and exposure time was 300 seconds per degree for all sets.  The scan width was 1o.   Data collection was 98.8% complete to 25o  in .  A total of  51575 merged reflections were collected covering the indices, -22<=h<=22, -33<=k<=33, -37<=l<=37, Rint = 0.2341.  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be P1  (No.2).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The contribution of about 18 disordered DCM solvent molecules per asymmetric unit to the diffraction pattern was removed with SQUEEZE (1563eV in voids). Two PF6- anions were found, where one of which appeared disordered.

The structure required several constraints and restraints. All phenyls were optimized with AFIX 66 … AFIX 0 loops. All complex motives P(Ph)2Ntol were related with the SAME command. All pyridine halves of 4,4’bipyridines were coupled with SAME. Pyridine moieties required FLAT restraints. Overall, an ISOR for carbons, nitrogen, and fluorine to prevent non-positive definite displacement matrices, and a DELU command including carbons, nitrogen and phosphorous were needed connecting the isotropic displacements along bonds. Some hydrogen atoms came slightly too close to each other which required their distances to be maintained with DFIX (since the data quality did not allow for a more detailed disorder model of phenyls and toluene) and 200 bad reflections were removed.

The structure exhibits a 2-dim network, where the Zn atoms of two complexes of the asymmetric unit are linked with one bypyridine (Zn1-N13 … N14-Zn4). Further contacts are established via bypyridine Zn2-N18 … N18’-Zn2’, Zn3-N15 … N17-Zn6, Zn5-N16 … N16’-Zn5’.

The structure is of appropriate quality and publishable.

Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level. Disorder omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C258 H228 Co12 F12 N18 P14 Se16 Zn6

Formula weight 	6604.89

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Triclinic

Space group 	P -1

Unit cell dimensions	a = 18.943(9) Å	a= 112.489(11)°.

b = 28.237(13) Å	b= 97.563(12)°.

c = 31.465(14) Å	g = 102.224(12)°.

Volume	14771(12) Å3

Z	2

Density (calculated)	1.485 Mg/m3

Absorption coefficient	3.234 mm-1

F(000)	6536

Crystal size	0.100 x 0.050 x 0.040 mm3

Theta range for data collection	0.828 to 25.027°.

Index ranges	-22<=h<=22, -32<=k<=32, -35<=l<=35

Reflections collected	51575

Independent reflections	51575 [R(int) = 0.2341]

Completeness to theta = 25.000°	98.8 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	51575 / 6364 / 2614

Goodness-of-fit on F2	1.012

Final R indices [I>2sigma(I)]	R1 = 0.1495, wR2 = 0.2958

R indices (all data)	R1 = 0.3551, wR2 = 0.4257

Largest diff. peak and hole	2.131 and -1.378 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,   Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. (2015). C71, 3–8

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A. (1997) MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland,.

Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.

(a) A. Spek, J. Appl. Cryst. (2003), 36, 7-13;

(b) P. van der Sluis & A. L. Spek, Acta Cryst. (1990). A46, 194-201.

(c) Spek, A. L. (2009). Acta. Cryst. D65, 148-155.

Farrugia, L. J. Appl. Cryst. 1997, 30, 565