Crystal Structure Report for D. Reed

by Paige Gannon & Werner Kaminsky

Sample Submitted by Spencer Smith

ID – SPW_2

X-ray Crystallography Laboratory

University of Washington

30. July 2026

Data Collection  started: 9/20/2022

Documentation finished: 9/27/2022

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A blue piece, measuring 0.10 x 0.08 x 0.08 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 30 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 99.4% complete to 25o in .  A total of 12869 reflections were collected covering the indices, -11<=h<=11, -15<=k<=15, -18<=l<=18.  6437 reflections were symmetry independent and the Rint = 0.0749 indicated that the data was of average quality.  Indexing and unit cell refinement indicated a triclinic lattice.  The space group was found to be P1 (No. 2).

The data appeared twinned (CELL_NOW) and after multi-domain integration (SAINT, SADABS within the APEX2 software package by Bruker) the data was merged utilizing twinabs.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The 1-dim polymeric structure contains a threefold disordered K(18-crown-6)+, a guanidinium cation, and a 2- charged partial Cu-O2(phenyl-SO3)2 complex which is completed by inversion symmetry, and a disordered methanol solvent (moiety formula C14 H8 Cu S2 O10, C12 H24 K O6, C H6 N3, C H4 O). The plymerization is facilitated through Cu and O5 on one end and O2/O4 the other.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the extended structure with thermal ellipsoids at the 50% probability level. Disorder omitted for clarity.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C28 H42 Cu K N3 O17 S2

Formula weight 	859.40

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Triclinic

Space group 	P -1

Unit cell dimensions	a = 10.0775(17) Å	a= 108.234(13)°.

b = 12.840(3) Å	b= 92.958(14)°.

c = 15.351(3) Å	g = 101.100(14)°.

Volume	1838.0(7) Å3

Z	2

Density (calculated)	1.553 Mg/m3

Absorption coefficient	0.898 mm-1

F(000)	894

Crystal size	0.100 x 0.080 x 0.080 mm3

Theta range for data collection	1.407 to 25.027°.

Index ranges	-11<=h<=11, -15<=k<=15, -18<=l<=18

Reflections collected	12869

Independent reflections	6437 [R(int) = 0.0749]

Completeness to theta = 25.000°	99.4 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	6437 / 448 / 611

Goodness-of-fit on F2	1.074

Final R indices [I>2sigma(I)]	R1 = 0.0716, wR2 = 0.1418

R indices (all data)	R1 = 0.1242, wR2 = 0.1632

Largest diff. peak and hole	0.765 and -0.616 e.Å-3

Sheldrick, G. M. (2005). CELL_NOW. University of Goettingen, Germany.

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

Sheldrick, G. M. (2007). TWINABS. University of Goettingen, Germany.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565