Crystal Structure Report for

Niels H. Anderson

Sample ID - ThiacumicinB

by Werner Kaminsky

X-ray Crystallography Laboratory

University of Washington

28. July 2026

Data Collection  started: 10/30/2002

Documentation finished: 11/1/2002

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A clear crystal plate size 0.55 x 0.29 x 0.05 mm was mounted on a glass capillary with oil.  Data was collected at -143oC with one set of exposure as follows:

Scan 		Type	Scanlength (deg.)  start (deg.)	     		     	

1	      	        	182.0					5.91	-160.5	160  	 0

Crystal-to-detector distance was 35 mm and exposure time was 30 seconds per degree for all sets.  The scan width was 1o.   Data collection was 59.6% complete to 24.14o in .  A total of  73632 partial and complete reflections were collected covering the indices, h = -14 to 14, k = -10 to 9,  l = -30 to 28.  4285 reflections were symmetry independent and the Rint = 0.144 indicated that the data was of slightly less than average quality (0.07).

Indexing and unit cell refinement indicated a monoclinic P lattice.  The space group was found to be P21 (No.4).

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S = exp(2B(sinq/)2)/ scale

S is calculated from the scale and the B factor is determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SIR97) produced a complete heavy atom phasing model consistent with the proposed structure.

All hydrogen atoms were located using a riding model. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares. Some atoms had to be constrained to obtain positive definite thermal parameter matrices. Two isopropanol and one acetic acid solvent were found.

Table 1.  Crystal data and structure refinement for shelxl.

Empirical formula 	C60 H94 Cl2 O22

Formula weight 	1238.25

Temperature 	130(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P 21 (No.4)

Unit cell dimensions	a = 13.485(5) Å	a= 90°.

b = 9.322(4) Å	b= 98.51(2)°.

c = 26.869(13) Å	g = 90°.

Volume	3340(2) Å3

Z	2

Density (calculated)	1.231 Mg/m3

Absorption coefficient	0.169 mm-1

F(000)	1328

Crystal size	0.55 x 0.29 x 0.05 mm3

Theta range for data collection	4.09 to 24.14°.

Index ranges	-14<=h<=14, -10<=k<=9, -30<=l<=28

Reflections collected	4285

Independent reflections	4285 [R(int) = 0.1440]

Completeness to theta = 24.14°	59.6 %

Absorption correction	HKL-SCALEPACK

Max. and min. transmission	0.9916 and 0.9129

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	4285 / 49 / 768

Goodness-of-fit on F2	0.775

Final R indices [I>2sigma(I)]	R1 = 0.0590, wR2 = 0.0958

R indices (all data)	R1 = 0.2280, wR2 = 0.1564

Absolute structure parameter	0.35(19)

Extinction coefficient	0.0067(13)

Largest diff. peak and hole	0.136 and -0.145 e.Å-3