Crystal Structure Report for B. Cossairt

by Werner Kaminsky

Sample Submitted by E. Miura-Stempel

ID – EMS III-002

X-ray Crystallography Laboratory

University of Washington

29. July 2026

Data Collection  started: 4/22/2024

Documentation finished: 4/30/2024

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A colorless needle, measuring 0.20 x 0.07 x 0.05 mm3 was mounted on a loop with oil.  Data was collected at -173oC on a Bruker APEX II single crystal X-ray diffractometer, Mo-radiation, equipped with a Miracol X-ray optical collimator.

Crystal-to-detector distance was 40 mm and exposure time was 15 seconds per frame for all sets.  The scan width was 0.5o.   Data collection was 100% complete to 25o in .  A total of  5122 reflections were collected covering the indices, -10<=h<=10, -10<=k<=10, -13<=l<=13.  771 reflections were symmetry independent and the Rint = 0.0702 indicated that the data was of average quality.  Indexing and unit cell refinement indicated an orthorhombic lattice.  The space group was found to be P c c n  (No. 56).

The data was integrated and scaled using SAINT, SADABS within the APEX2 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL. Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were refined isotropically and all non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The data appeared twinned via 180 degrees about (1 1 0)  and after introducing a merohedral twin law of a 2-fold rotation about (1 1 0) the R1 dropped from 14% to 3%.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	H4 N2 O3

Formula weight 	80.05

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Orthorhombic

Space group 	P c c n

Unit cell dimensions	a = 7.9249(9) Å	a= 90°.

b = 7.9270(9) Å	b= 90°.

c = 9.8035(11) Å	g = 90°.

Volume	615.86(12) Å3

Z	8

Density (calculated)	1.727 Mg/m3

Absorption coefficient	0.181 mm-1

F(000)	336

Crystal size	0.200 x 0.070 x 0.050 mm3

Theta range for data collection	2.077 to 28.332°.

Index ranges	-10<=h<=10, -10<=k<=10, -13<=l<=13

Reflections collected	5122

Independent reflections	771 [R(int) = 0.0702]

Completeness to theta = 25.242°	100.0 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	771 / 0 / 65

Goodness-of-fit on F2	1.093

Final R indices [I>2sigma(I)]	R1 = 0.0303, wR2 = 0.0692

R indices (all data)	R1 = 0.0399, wR2 = 0.0759

Largest diff. peak and hole	0.230 and -0.268 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

(a) Sheldrick GM. (2008) A short history of SHELX. Acta Cryst. A64, 112-122.

(b) Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,  Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. C71, 3–8

Waasmaier, D.; Kirfel, A. (1995) New Analytical Scattering Factor Functions for Free Atoms and Ions. Acta Crystallographica A., 51, 416-430.

Farrugia LJ. (1997) Ortep-3 for Windows. Journal of Applied Crystallography, 30, 565