Crystal Structure Report for

K. S. Singh

by Werner Kaminsky

Sample ID – kss14A

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 3/4/2020

Documentation finished: 3/6/2020

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

An orange prism, measuring 0.50 x 0.20 x 0.17 mm3 was mounted on a loop with oil.  Data was collected at -143oC on a Nonius Kappa CCD FR590 single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 30 mm and exposure time was 20 seconds per degree for all sets.  The scan width was 1o.   Data collection was 99.9% complete to 25o  in .  A total of  13300 merged reflections were collected covering the indices, -14<=h<=12, -21<=k<=22, -14<=l<=12.  4106 reflections were symmetry independent and the Rint = 0.0456 indicated that the data was excellent.  Indexing and unit cell refinement indicated a primitive orthorhombic lattice.  The space group was found to be P n a 21  (No. 33).

The data was integrated and scaled using SAINT, SADABS within the APEX3 software package by Bruker.

Solution by direct methods (SHELXT or SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C16 H22 Cl F6 N2 P Ru

Formula weight 	523.84

Temperature 	100(2) K

Wavelength 	0.71073 Å

Crystal system 	Orthorhombic

Space group 	P n a 21

Unit cell dimensions	a = 10.6879(8) Å	a= 90°.

b = 17.0868(15) Å	b= 90°.

c = 10.8340(10) Å	g = 90°.

Volume	1978.5(3) Å3

Z	4

Density (calculated)	1.759 Mg/m3

Absorption coefficient	1.066 mm-1

F(000)	1048

Crystal size	0.500 x 0.200 x 0.170 mm3

Theta range for data collection	2.226 to 28.294°.

Index ranges	-14<=h<=12, -21<=k<=22, -14<=l<=12

Reflections collected	13300

Independent reflections	4106 [R(int) = 0.0456]

Completeness to theta = 25.000°	99.9 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	4106 / 1 / 247

Goodness-of-fit on F2	1.054

Final R indices [I>2sigma(I)]	R1 = 0.0315, wR2 = 0.0716

R indices (all data)	R1 = 0.0394, wR2 = 0.0754

Absolute structure parameter	-0.03(2)

Largest diff. peak and hole	1.414 and -0.817 e.Å-3

Bruker (2007) APEX2 (Version 2.1-4), SAINT (version 7.34A), SADABS (version 2007/4), BrukerAXS Inc, Madison, Wisconsin, USA.

Sheldrick GM. (2015) SHELXT - Integrated space-group and crystal-structure determination. Acta Cryst. A71, 3-8.

(a) Altomare A, Burla C, Camalli M, Cascarano G L, Giacovazzo C, Guagliardi A,   Moliterni AGG, Polidori G, Spagna R. (1999) SIR97: a new tool for crystal structure determination and refinement Journal of Applied Crystallography, 32, 115-119.

(b) Altomare A, Cascarano G L, Giacovazzo C, Guagliardi A. (1993) Completion and refinement of crystal structures with SIR 92. Journal of Applied Crystallography, 26, 343-350.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2015) Crystal structure refinement with SHELXL. Acta Cryst. (2015). C71, 3–8

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A. (1997) MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland,.

Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.

Farrugia, L. J. Appl. Cryst. 1997, 30, 565