Crystal Structure Report for

Rupam Dinda

by Werner Kaminsky

ID – MM_15

X-ray Crystallography Laboratory

University of Washington

31. July 2026

Data Collection  started: 7/27/2017

Documentation finished: 7/31/2017

Data backup accessibly through http:

URL: http://cad4.cpac.washington.edu/structures

W.Kaminsky: tel. (206) 543 7585

(x-ray lab):     tel. (206) 543 0210

Email: kaminsky@chem.washington.edu

Understanding crystallographic information is not trivial. If you discuss a structure in a publication, please let the crystallographers proof-read the final draft of that manuscript before you submit it.

For many structures it is true that crystallography is an intellectual performance (like synthesis or elucidation of mechanisms) and not a routine job. In these cases, particularly when the structures contribute significantly to the content of the publication, the crystallographers should be co-authors. Do not think this is not necessary because you pay for structures.

A light brown prism, measuring 0.20 x 0.09 x 0.05 mm3 was mounted on a loop with oil.  Data was collected at -143oC on a Nonius Kappa CCD FR590 single crystal X-ray diffractometer, Mo-radiation.

Crystal-to-detector distance was 30 mm and exposure time was 60 seconds per degree for all sets.  The scan width was 2o.   Data collection was 99.9% complete to 25.3o and 94.3% complete to 25o  in .  A total of  25427 partial and complete reflections were collected, covering the indices, -16<=h<=16, -12<=k<=13, -18<=l<=18.  12420 reflections were symmetry independent with an alevated Rint = 0.1532 which reflects a rather small sample size. Indexing and unit cell refinement indicated a primitive monoclinic lattice.  The space group was found to be P 21/c  (No. 14).

The data was integrated and scaled using hkl-SCALEPACK. This program applies a multiplicative correction factor (S) to the observed intensities (I) and has the following form:

S is calculated from the scale and the B factor determined for each frame and is then applied to I to give the corrected intensity (Icorr).

Solution by direct methods (SHELXS, SIR97) produced a complete heavy atom phasing model consistent with the proposed structure. The structure was completed by difference Fourier synthesis with SHELXL97., Scattering factors are from Waasmair and Kirfel. Hydrogen atoms were placed in geometrically idealised positions and constrained to ride on their parent atoms with C---H distances in the range 0.95-1.00 Angstrom. Isotropic thermal parameters Ueq were fixed such that they were 1.2Ueq of their parent atom Ueq for CH's and 1.5Ueq of their parent atom Ueq in case of methyl groups. All non-hydrogen atoms were refined anisotropically by full-matrix least-squares.

The structure is of high quality and ready for publication. Table 1 summarizes the data collection details. Figure 1 shows an ORTEP of the asymmetric unit.

Figure 1. ORTEP of the structure with thermal ellipsoids at the 50% probability level.

Figure 2. Dimeric packing pattern via Cu1 – O1 interactions perpendicular to the molecular planes.

Table 1: Crystallographic data for the structures provided.

Empirical formula 	C54 H52 Cu2 N8 O6

Formula weight 	1036.11

Temperature 	130(2) K

Wavelength 	0.71073 Å

Crystal system 	Monoclinic

Space group 	P 21/c

Unit cell dimensions	a = 14.0290(17) Å	a= 90°.

b = 11.4323(14) Å	b= 108.441(8)°.

c = 15.227(3) Å	g = 90°.

Volume	2316.8(6) Å3

Z	2

Density (calculated)	1.485 Mg/m3

Absorption coefficient	0.981 mm-1

F(000)	1076

Crystal size	0.200 x 0.090 x 0.050 mm3

Theta range for data collection	2.979 to 25.348°.

Index ranges	-16<=h<=16, -12<=k<=13, -18<=l<=18

Reflections collected	7264

Independent reflections	4192 [R(int) = 0.1532]

Completeness to theta = 25.000°	99.0 %

Refinement method	Full-matrix least-squares on F2

Data / restraints / parameters	4192 / 0 / 319

Goodness-of-fit on F2	0.884

Final R indices [I>2sigma(I)]	R1 = 0.0618, wR2 = 0.1052

R indices (all data)	R1 = 0.1990, wR2 = 0.1481

Largest diff. peak and hole	0.608 and -0.827 e.Å-3

Otwinowsky, Z.; Minor, W. In: Methods in Enzymology; Carter, C. W., Jr., Sweet, R. M., Eds.; Academic Press: New York, 1997, 276, 307-326.

(a) Altomare, A.; Burla, C.; Camalli M.; Cascarano L.; Giacovazzo C.; Guagliardi A.;   Moliterni A.G.G.; Polidori G.; Spagna R. J. Appl. Cryst. 1999, 32, 115-119; (b) Altomare, A., Cascarano, G., Giacovazzo, C., Guagliardi, A., J. Appl. Cryst. 1993, 26, 343.

(a) Sheldrick GM. (1997) SHELXL-97, Program for the Refinement of Crystal Structures. University of Göttingen, Germany.

(b) Sheldrick GM. (2013) Crystal structure refinement with SHELXL. Acta Cryst. (2015). C71, 3-8.

Mackay, S.; Edwards, C.; Henderson, A.; Gilmore, C.; Stewart, N.; Shankland, K.; Donald, A.; MaXus: a computer program for the solution and refinement of crystal structures from diffraction data. University of Glasgow, Scotland, 1997.

Waasmaier, D.; Kirfel, A. Acta Crystallogr. A. 1995, 51, 416.

Farrugia, L. J. Appl. Cryst. 1997, 30, 565